SilentFiber Wave Baffle Installation Guide:

Step 1: Locate the area in which your baffles will be hung

Step 2: Clean the provided cables with a damp rag or washcloth

Step 3: Run 3 cables through the ceiling grid (for 1 baffle), placed approximately 4 feet apart

Step 4: Starting with the center hanger, hold the baffle up to the bottom of the cable and connect center hanger with center cable

Step 5: Pull cable through the hanger until it is tight and at the appropriate height

Step 6: Repeat steps 3 & 4 for the second and third cable/hanger

Step 7: Using a leveling tool, check to make sure your baffle is even

Step 8: to loosen or detach the cable from the hanger, pinch in the plastic sides of the hanger

For more information:

info@soundacousticsolutions.com

www.soundacousticsolutions.com

Call toll free: +1 (877) 399-9697

Follow us on social media: